# EV Supply Chain ABM+SD Model

# Simulation Results Report

Focus region: UK  |  Simulation horizon: 260 weeks (5 years)  |  Seed: 42  |  Scenarios: 34

# 1. Executive Summary

This report presents results from the UK-focused EV supply-chain Agent-Based Model (ABM) coupled with a System Dynamics (SD) layer. The model simulates five tiers of the EV supply chain — critical minerals, cell manufacturing, Tier-1 sub-systems, OEM assembly, and consumer markets — over a five-year horizon (260 weeks) starting from 2023 calibration anchors. Thirty-four scenarios are evaluated, covering a baseline, nine geopolitical and supply disruption shocks, and three policy packages applied to each shock.

Key findings:

The UK OEM is highly resilient to global mineral supply shocks due to its small market share (~2.4% of global cell demand) and multi-tier safety stocks.

Only direct UK-specific disruptions — Ukraine harness plant closure and UK supply-chain friction — produce measurable production losses (0–35.6 k vehicles).

The Full Industrial Strategy package (DRIVE35) raises mean OEM production by 17% under UK friction, reduces cumulative loss from 35,583 to 2,444 vehicles, and shortens recovery from week 128 to week 106.

Mineral prices rise measurably under all supply shocks; the LFP chemistry substitution feedback loop partially buffers cobalt scarcity.

All 647 automated checks across four test suites pass with zero failures.

# 2. Baseline Calibration

The model is calibrated to 2023 published data from IEA, SMMT, USGS, and BloombergNEF. Table 1 compares week-0 model outputs against calibration targets.

Table 1: Baseline calibration against 2023 anchors

| Metric | Model (Week 0) | Calibration Target | Status |
| --- | --- | --- | --- |
| UK OEM annual production | 184.8 k veh/yr | 175 k veh/yr (JLR 2023) | PASS |
| UK cell demand | 20.1 GWh/yr | 20.0 GWh/yr (IEA 2023) | PASS |
| Market demand (UK) | 20.1 GWh/yr | 20.0 GWh/yr | PASS |
| Initial battery price index | 1.001 | 1.000 ± 0.05 | PASS |
| LFP share (initial) | 39.8% | ~40% (BloombergNEF 2023) | PASS |
| Cobalt stock buffer | 6.0 wk | 6 wk (USGS MCS target) | PASS |
| REE stock buffer | 8.9 wk | 8 wk (strategic reserve) | PASS |
| SiC stock buffer | 12.6 wk | 12 wk (26-wk lead time) | PASS |
| Bullwhip index | 2.42 | > 1.0 (amplification expected) | PASS |

Baseline dynamics over the five-year horizon:

OEM production grows from 3.55 k veh/wk (week 0) to 15.49 k veh/wk (week 260), consistent with the IEA 28%/yr demand growth trajectory.

Market demand grows at an implied 28.0%/yr, within the IEA 15–35% range.

LFP share rises from 39.8% to 51.4% as battery chemistry shifts driven by cobalt price signals and cost advantages.

The composite battery price signal ranges from 1.00 to 1.43 (index, baseline = 1.0), reflecting progressive raw-material scarcity as demand outpaces supply growth.

Mean mineral stock levels: cobalt 5.3 wk, graphite 3.0 wk, REE 8.5 wk, SiC wafer 12.0 wk — all within ±50% of configured safety targets.

# 3. Shock Scenario Results

Table 2 summarises all nine base shock scenarios ranked by cumulative OEM production loss vs the baseline over the 260-week horizon.

Table 2: Shock scenario performance metrics (260-week, seed 42)

| Scenario | Mean OEM (k veh/wk) | Cum. Loss (k veh) | Peak Price Signal | Max Backlog (k veh) | Min Cobalt Stock (wk) |
| --- | --- | --- | --- | --- | --- |
| UK Supply Chain Friction | 8.39 | 35.6 | 1.412 | 138 | 4.44 |
| US–China Tariff Shock | 8.53 | 0.5 | 1.434 | 124 | 4.67 |
| China REE Export Restriction | 8.53 | 0.3 | 1.434 | 124 | 4.45 |
| China Graphite Disruption | 8.53 | 0.3 | 1.434 | 124 | 4.53 |
| CATL Disruption (China) | 8.53 | 0.3 | 1.434 | 124 | 4.44 |
| Compound Shock (Cobalt + Harness) | 8.53 | 0.2 | 1.433 | 124 | 4.38 |
| DRC Cobalt Supply Shock | 8.53 | 0.2 | 1.433 | 124 | 4.38 |
| SiC Wafer Bottleneck | 8.53 | 0.2 | 1.433 | 124 | 4.45 |
| Ukraine Harness Plant Closure | 8.53 | 0.0 | 1.433 | 124 | 4.45 |

## 3.1 Scenario Narratives

UK Supply Chain Friction

The most damaging scenario for UK production. A sustained 10% severity shock to uk_oem (weeks 4–56) and 8% to the harness supplier (weeks 4–30) produces a cumulative loss of 35,583 vehicles — 94× larger than the next worst scenario. Mean OEM output falls from 8.53 to 8.39 k veh/wk (−1.6%), with a maximum backlog of 138 k vehicles. The shock persists because it directly impairs the UK-specific OEM and its domestic harness supplier.

US–China Tariff Shock

Sustained shocks to CATL (−30%), China graphite (−35%), and China REE (−30%) from week 26–260/130 compound gradually over the five-year horizon. Cumulative loss is 0.45 k vehicles — modest because the UK OEM's small global footprint means it can continue sourcing from non-China suppliers.

DRC Cobalt Supply Shock

A 50% severity shock to DRC cobalt agents (weeks 26–52) reduces cobalt stock by 0.077 wk during the peak depletion window (weeks 34–60). Cobalt price rises modestly (+0.2 pp above baseline). The LFP chemistry substitution loop partially absorbs demand, limiting OEM production loss to 0.22 k vehicles.

Ukraine Harness Plant Closure

Despite an 80% severity shock (weeks 4–12) followed by 35% (weeks 12–36) to the harness Tier-1 agent, cumulative OEM production loss is negligible (0 k vehicles). The harness agent has substantial spare capacity relative to UK demand, so even 20% of capacity exceeds the weekly demand of ~3.6 k units. OEM backlog rises slightly during the shock period.

China Graphite Disruption

A 45% severity shock to graphite_chn (weeks 8–60) depletes graphite stock by 0.088 wk at peak (weeks 12–60). Graphite price rises above baseline. Cumulative loss is 0.27 k vehicles over 260 weeks.

## 3.2 Shock Propagation Chain

Table 3 traces the signal at each tier for the three mineral shocks with verified propagation.

Table 3: Mineral stock and price propagation during shock windows

| Scenario | Mineral | Stock (base->shock) wk | Stock Change | Price (base->shock) | Price Change |
| --- | --- | --- | --- | --- | --- |
| DRC Cobalt Supply Shock | Cobalt | 5.935 -> 5.857 | -1.3% | 1.2404 -> 1.2437 | +0.26% |
| China Graphite Disruption | Graphite | 3.947 -> 3.885 | -1.6% | 1.2147 -> 1.2207 | +0.49% |
| China REE Export Restriction | Ree | 8.811 -> 8.687 | -1.4% | 1.1942 -> 1.2012 | +0.58% |

# 4. Policy Intervention Results

Four policy packages derived from the UK Advanced Manufacturing Sector Plan and DRIVE35 framework are evaluated. Table 4 compares all three packages against the no-policy baseline for the UK Supply Chain Friction scenario — the most policy-relevant case.

Table 4: Policy effectiveness under UK Supply Chain Friction scenario (260 weeks)

| Policy Package | Mean OEM (k/wk) | Cum. Loss (k veh) | Recovery Week | OEM Uplift vs No-Policy |
| --- | --- | --- | --- | --- |
| No policy | 8.395 | 35.6 | 128 | — |
| Tier-1 Resilience | 9.278 | 7.2 | 112 | +10.5% |
| Critical Minerals | 8.409 | 32.5 | 129 | +0.2% |
| Full Industrial Strategy | 9.823 | 2.4 | 106 | +17.0% |

## 4.1 Package-Level Analysis

Battery Sovereignty Package

Doubles AESC UK cell weekly capacity (×2.25), accelerates capacity growth (×1.65), and reduces imported cell concentration risk. The SD policy layer activates a 13-week linear ramp, cutting the CAPEX trigger threshold by 8% and raising build speed by 20%. The package contributes primarily to the Full Strategy uplift rather than acting as a standalone supply-chain defence for the UK OEM.

Tier-1 Resilience Package

Raises harness safety stock by 1.45× and other Tier-1 components by 1.25×. Recovery rate improves by 30% and capacity growth by 20% across all four Tier-1 agents. Under UK friction, this package alone reduces cumulative loss by 80% (35.6 → 7.2 k vehicles) and accelerates recovery by 16 weeks. This is the single most effective package for UK domestic resilience.

Critical Minerals Security Package

Injects strategic buffer stock (cobalt +2 wk, REE +3 wk, SiC +3 wk) and activates recycling/urban-mining pathways (outflow reduction 5–15% per mineral). Under UK friction, the standalone minerals package has limited effect (+0.2% mean OEM) because the UK friction scenario is constrained at the OEM/Tier-1 level, not at the mineral level. However, mineral stocks are substantially higher in all mineral-policy scenarios (cobalt: 4.4 → 7.6 wk).

Full Industrial Strategy (DRIVE35)

Combines all three packages with an additional 10% overlay on capacity growth and recovery rates across all agents. Reduces UK friction cumulative loss by 93% (35,583 → 2,444 vehicles), raises mean OEM production by 17%, and shortens recovery to week 106 (vs 128 without policy). This represents the upper bound of achievable resilience within the model's parameterisation.

## 4.2 Full Strategy — Cross-Scenario Comparison

Table 5: Full Industrial Strategy impact across all shock scenarios

| Scenario | No Policy (k/wk) | Full Policy (k/wk) | Uplift | Recovery (No Policy) | Recovery (Full Policy) |
| --- | --- | --- | --- | --- | --- |
| UK Supply Chain Friction | 8.395 | 9.823 | +17.0% | 128 | 106 |
| China REE Export Restriction | 8.530 | 9.879 | +15.8% | 128 | 103 |
| China Graphite Disruption | 8.532 | 9.878 | +15.8% | 128 | 103 |
| CATL Disruption (China) | 8.532 | 9.880 | +15.8% | 128 | 103 |
| Compound Shock (Cobalt + Harness) | 8.531 | 9.880 | +15.8% | 128 | 103 |
| DRC Cobalt Supply Shock | 8.531 | 9.880 | +15.8% | 128 | 103 |
| SiC Wafer Bottleneck | 8.531 | 9.880 | +15.8% | 128 | 103 |
| Ukraine Harness Plant Closure | 8.532 | 9.880 | +15.8% | 128 | 103 |

# 5. Model Validation

The model was validated through four complementary test suites. Table 6 summarises the validation scorecard.

Table 6: Validation scorecard

| Suite | File | Total Checks | Pass | Fail |
| --- | --- | --- | --- | --- |
| SD Layer Unit Tests | test_sd_model.py | 32 | 32 | 0 |
| SD Policy Tests | test_sd_policy.py | 55 | 55 | 0 |
| ABM & Hybrid Tests | test_abm.py | 84 | 84 | 0 |
| Full Model Validation | validate_model.py | 476 | 476 | 0 |
| TOTAL | — | 647 | 647 | 0 |

## 5.1 Validation Categories

static (180)

Configuration integrity: cell market shares sum to 1.0, OEM targets match global anchor, all shock targets resolvable to named agents.

invariants (170)

Numeric stability: finite values, non-negative stocks and flows, bounded LFP share [0.20–0.80], bounded price indices [0.5–6.0].

run (34)

All 34 scenarios simulate to completion without exception.

baseline (5)

Week-0 outputs within ±15% of 2023 calibration anchors for OEM production, cell output, demand, and price signal.

scenario (14)

Face-validity: UK friction reduces OEM output; mineral shocks deplete stocks and raise prices; compound shock ≥ largest single shock.

propagation (12)

Full chain tracing: DRC cobalt → stock falls → price rises → LFP shift; graphite shock → stock/price; CATL shock → cell production falls; UK friction → OEM production falls → backlog rises; price recovery after shock ends.

policy (9)

Each policy package raises mean OEM production vs same-shock no-policy; Full Strategy shortens recovery; minerals policy raises cobalt stock during DRC shock.

dynamics (7)

System dynamics: demand grows at IEA-range rate; LFP bounded; bullwhip > 1; cell utilisation non-trivial.

consistency (9)

Cross-layer consistency: mineral stocks near target; no cell stockout; OEM production grows 260 weeks; price signal > 1.0 in steady state.

real_timeseries (2)

ONS/SMMT indexed comparison: baseline and UK friction MAE reported over 34 months (2023–2025). Note: model tracks UK EV OEM output; observed series is total UK car registrations — structural difference limits direct MAE interpretation.

outputs (34)

All 34 scenario CSV files present and schema-consistent with fresh model output.

## 5.2 Historical Time-Series Comparison

The model baseline was compared against the ONS/SMMT UK new vehicle registration index (seasonally adjusted, 2023 average = 100) over 34 months (2023-01 to 2025-10). The indexed MAE is 51.0 index points (MAPE = 64.2%). This large error reflects a structural mismatch: the observed series covers total UK car registrations (~75,846 vehicles/month) while the model tracks UK EV-oriented OEM output only (~18,377 k vehicles/month). A direct level comparison is therefore not appropriate; the indexed comparison confirms the model captures the directional growth trend.

# 6. Model Limitations and Caveats

UK focus and global supply chain

The UK OEM represents ~2.4% of global cell demand. Global mineral shocks produce modest price and stock signals because the UK is a price-taker, not a price-setter, in global markets. Larger effects would be visible in a global-focus run.

Agent heterogeneity vs real-world complexity

Each supply-chain tier is represented by a small number of archetypal agents (9 cell manufacturers, 4 Tier-1 types, 7 OEMs). Real-world heterogeneity and firm-level relationships are necessarily abstracted.

Deterministic policy representation

Policy packages apply step changes (with 13-week linear ramp) to agent parameters. In practice, policy effects are uncertain, time-lagged, and conditional on complementary investments.

No financial contagion

The model does not represent credit constraints, equity markets, or insolvency cascades that could amplify supply shocks in reality.

Exogenous demand growth

Market demand grows at a fixed IEA-calibrated rate independent of vehicle price or income effects. Consumer behaviour responses to price shocks are captured only through the price-elasticity parameter.

Historical MAE caveat

The real time-series comparison covers total UK car registrations (ICE + EV) against the model's UK EV OEM output. This is structurally incomparable and the MAE should be interpreted as a trend-direction check, not an accuracy measure.